SeismoGuard Real-Time Seismic Protection System ───────────────────────────────── Complete Project Documentation

| Hardware ESP32 Dev Board MPU6050 Accelerometer I2C LCD 16x2 Active Buzzer + LED | Software Arduino IDE (ESP32 Firmware) Python 3 (UDP Receiver) XAMPP / MySQL (Database) PHP + HTML/JS (Dashboard) |
| --- | --- |

| WiFi UDP Port 4210 | MySQL DB quakesense | 3s Refresh Live Dashboard | 3 Levels NORMAL/WARN/QUAKE |
| --- | --- | --- | --- |

Version 1.0  —  April 2026

# 1.  Project Overview

## What is SeismoGuard?

SeismoGuard is a full-stack IoT earthquake detection and alerting system. An ESP32 microcontroller continuously reads vibration data from an MPU6050 accelerometer, applies a low-pass filter to eliminate noise, and classifies the result into three alert levels: NORMAL, WARNING, and EARTHQUAKE!.

When vibration is detected above a threshold, the system immediately triggers a local alarm (buzzer + LED), displays the status on an LCD screen, and simultaneously transmits the data wirelessly via UDP over WiFi to a PC. On the PC, a Python script captures this data and stores WARNING and EARTHQUAKE events into a MySQL database. A live PHP/HTML dashboard at localhost refreshes every 3 seconds to show the current status, historical event log, and statistics.

## System Flow Diagram

+---------------------------+

¦ ESP32 + MPU6050 Sensor ¦

¦ Reads vibration every ¦

¦ 100ms, classifies it ¦

+------------+--------------+

¦

¦ WiFi UDP Packet (port 4210)

¦ Format: "STATUS ¦ D:value"

v

+---------------------------+

¦ Python receiver.py ¦ <-- runs on your PC

¦ Listens on UDP 4210 ¦

¦ Saves WARNING/QUAKE ¦

+------------+--------------+

¦

¦ SQL INSERT

v

+---------------------------+

¦ MySQL Database ¦

¦ quakesense.seismic_data ¦

+------------+--------------+

¦

¦ PHP api.php (JSON response)

v

+---------------------------+

¦ index.html Dashboard ¦

¦ Auto-refreshes every 3s ¦

¦ http://localhost/ ¦

¦ seismoguard/ ¦

+---------------------------+

## Alert Level Definitions

| Alert Level | quakeCounter Value | System Response |
| --- | --- | --- |
| NORMAL | 0 to 2 | LCD shows NORMAL. LED off. UDP sent. Not saved to DB. |
| WARNING | 3 to 6 | LCD shows WARNING. UDP sent. Saved to database. |
| EARTHQUAKE! | Greater than 6 | Buzzer beeps 5 times. LED on. Full alert on LCD. Saved to DB. Dashboard flashes red. |

## Complete File Reference

| File | Location | Role |
| --- | --- | --- |
| earthquake-AI.ino | Arduino IDE Project | ESP32 firmware — sensor reading, detection, LCD, buzzer, UDP |
| receiver.py | C:\xampp\htdocs\project\ | Python UDP listener — parses packets, saves to MySQL |
| api.php | C:\xampp\htdocs\seismoguard\ | PHP backend — reads DB, returns JSON |
| index.html | C:\xampp\htdocs\seismoguard\ | Live web dashboard — fetches api.php every 3 seconds |
| db_setup.sql | Run in phpMyAdmin | Creates the quakesense database and seismic_data table |

# 2.  Hardware — Components & Wiring

## Components Required

| Component | Qty | Description |
| --- | --- | --- |
| ESP32 Dev Board | 1 | Main controller — has built-in WiFi and Bluetooth. Any 30-pin or 38-pin ESP32 works. |
| MPU6050 | 1 | 6-axis IMU — provides 3-axis accelerometer + 3-axis gyroscope over I2C. |
| I2C LCD 16x2 | 1 | 16 column x 2 row LCD with I2C backpack. Default address 0x27. |
| Active Buzzer | 1 | Generates sound when HIGH signal applied. Must be active (not passive). |
| Red LED | 1 | Standard 5mm red LED for visual alarm. |
| 220 ohm resistor | 1 | Current limiter for the LED. |
| Jumper wires | 15+ | Male-to-male and male-to-female for GPIO connections. |
| Breadboard | 1 | Optional — for prototyping connections cleanly. |
| USB cable | 1 | Micro-USB or USB-C depending on your ESP32 board — for power and flashing. |

# STEP 1  —  Wire the MPU6050 Accelerometer

The MPU6050 communicates via I2C. Connect it to the ESP32's default I2C pins. Both SDA and SCL will be shared with the LCD on the same bus.

| MPU6050 Pin | Connect to ESP32 |
| --- | --- |
| VCC | 3.3V pin |
| GND | GND pin |
| SDA | GPIO 21 (ESP32 default SDA) |
| SCL | GPIO 22 (ESP32 default SCL) |
| AD0 | GND (this sets the I2C address to 0x68) |
| INT | Not connected (not used in this project) |

⚠ I2C Address Note: Connecting AD0 to GND gives the MPU6050 address 0x68. Connecting it to 3.3V would give 0x69. This project uses 0x68.

# STEP 2  —  Wire the I2C LCD 16x2

The LCD uses an I2C backpack module which reduces the pin count to just 4 wires. It shares the I2C bus with the MPU6050.

| LCD (I2C Backpack) Pin | Connect to ESP32 |
| --- | --- |
| VCC | 5V pin (or 3.3V if your module supports it) |
| GND | GND pin |
| SDA | GPIO 21 (same as MPU6050 SDA — shared bus) |
| SCL | GPIO 22 (same as MPU6050 SCL — shared bus) |

ℹ LCD I2C Address: The LCD's I2C address is 0x27 on most modules. If your LCD shows nothing after wiring, try changing 0x27 to 0x3F in the Arduino code.

# STEP 3  —  Wire the Buzzer

| Buzzer Pin | Connect to ESP32 |
| --- | --- |
| + (positive, longer leg) | GPIO 13 |
| - (negative, shorter leg) | GND pin |

# STEP 4  —  Wire the LED

| LED Pin | Connect to ESP32 |
| --- | --- |
| Anode (longer leg) | GPIO 2 — via 220 ohm resistor in series |
| Cathode (shorter leg) | GND pin |

⚠ Resistor Required: Always use a 220-330 ohm resistor in series with the LED anode. Without it, you risk burning out the LED or the ESP32 GPIO pin.

## Complete Wiring Summary

| Component | ESP32 Pin | Notes |
| --- | --- | --- |
| MPU6050 VCC | 3.3V | Do not connect to 5V — will damage the chip |
| MPU6050 GND | GND |  |
| MPU6050 SDA | GPIO 21 | Shared I2C bus |
| MPU6050 SCL | GPIO 22 | Shared I2C bus |
| MPU6050 AD0 | GND | Sets address to 0x68 |
| LCD VCC | 5V |  |
| LCD GND | GND |  |
| LCD SDA | GPIO 21 | Shared with MPU6050 |
| LCD SCL | GPIO 22 | Shared with MPU6050 |
| Buzzer + | GPIO 13 | Active buzzer |
| Buzzer - | GND |  |
| LED Anode | GPIO 2 | Via 220 ohm resistor |
| LED Cathode | GND |  |

# 3.  Arduino IDE Setup

# STEP 5  —  Install Arduino IDE

- Go to https://www.arduino.cc/en/software

- Download and install the version for your OS (Windows/Mac/Linux).

- Launch Arduino IDE after installation.

# STEP 6  —  Add ESP32 Board Support

- Click File > Preferences (or Arduino > Preferences on Mac).

- Find the field labelled Additional boards manager URLs.

- Paste the following URL into that field:

https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Click OK to close Preferences.

- Go to Tools > Board > Boards Manager.

- Search for esp32 in the search box.

- Find the entry by Espressif Systems and click Install.

- Wait for the installation to complete (may take several minutes).

- Go to Tools > Board > ESP32 Arduino > ESP32 Dev Module.

✔ Board Selected: After this step, all future uploads will target the ESP32. The board name shown in the bottom bar of Arduino IDE should say ESP32 Dev Module.

# STEP 7  —  Install Required Libraries

Go to Tools > Manage Libraries (or Sketch > Include Library > Manage Libraries). Search for and install each library below:

| Library Name | Details / Author |
| --- | --- |
| LiquidCrystal I2C | Author: Frank de Brabander — IMPORTANT: install this specific version, not the AVR version |
| Wire | Built-in — comes with Arduino IDE, no install needed |
| WiFi | Built-in — included with the ESP32 board package |
| WiFiUdp | Built-in — included with the ESP32 board package |

⚠ Critical Library Warning: Do NOT install the LiquidCrystal I2C library labelled for AVR architecture. It is incompatible with ESP32 and will cause compile errors. You must install the one by Frank de Brabander.

# STEP 8  —  Select Port and Board Settings

- Connect your ESP32 to your PC via USB cable.

- Go to Tools > Port and select the COM port for your ESP32 (e.g. COM3 or COM6).

- Go to Tools > Upload Speed and set it to 115200.

- Go to Tools > Board and confirm ESP32 Dev Module is selected.

If you do not see a COM port, install the CP2102 or CH340 USB driver depending on your ESP32 board's USB chip.

# 4.  Arduino Code (ESP32 Firmware)

## Configuration — Before You Upload

Open a new sketch in Arduino IDE and paste the full code. Before uploading, you MUST change the following four values at the top of the code to match your setup:

| Variable | What to change it to |
| --- | --- |
| ssid | Your WiFi network name (exactly as it appears, case-sensitive) |
| password | Your WiFi password |
| udpAddress | Your PC's local IP address — find it by running ipconfig in Command Prompt, look for IPv4 Address under your WiFi adapter. Example: 192.168.0.101 |
| udpPort | Leave as 4210 — must match receiver.py |
| threshold | Leave as 0.8 to start. Adjust later if needed. |

## Full Arduino Code

#include <WiFi.h>

#include <WiFiUdp.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

// ---- WIFI ----

const char* ssid = "YourWiFiName"; // <-- CHANGE THIS

const char* password = "YourWiFiPassword"; // <-- CHANGE THIS

// ---- UDP ----

const char* udpAddress = "192.168.0.101"; // <-- YOUR PC IP

const int udpPort = 4210;

WiFiUDP udp;

// ---- MPU6050 ----

const int MPU = 0x68;

// ---- LCD ----

LiquidCrystal_I2C lcd(0x27, 16, 2);

// ---- OUTPUT PINS ----

const int ledPin = 2;

const int buzzerPin = 13;

// ---- DETECTION VARIABLES ----

float accMagnitude = 0;

float prevMagnitude = 0;

float diff = 0;

float filteredDiff = 0;

int quakeCounter = 0;

float threshold = 0.8; // sensitivity

void setup() {

Serial.begin(115200);

delay(1000);

Wire.begin();

pinMode(ledPin, OUTPUT);

pinMode(buzzerPin, OUTPUT);

lcd.init();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("SeismoGuard");

lcd.setCursor(0, 1);

lcd.print("Starting...");

WiFi.begin(ssid, password);

int timeout = 0;

while (WiFi.status() != WL_CONNECTED && timeout < 20) {

delay(500);

Serial.print(".");

timeout++;

}

lcd.clear();

if (WiFi.status() == WL_CONNECTED) {

lcd.print("WiFi Connected");

} else {

lcd.print("WiFi Failed");

}

delay(1500);

lcd.clear();

Wire.beginTransmission(MPU);

Wire.write(0x6B);

Wire.write(0);

Wire.endTransmission(true);

udp.begin(udpPort);

}

void readMPU(float &ax, float &ay, float &az) {

Wire.beginTransmission(MPU);

Wire.write(0x3B);

Wire.endTransmission(false);

int bytes = Wire.requestFrom(MPU, 14, true);

if (bytes != 14) return;

ax = (Wire.read() << 8 ¦ Wire.read()) / 16384.0;

ay = (Wire.read() << 8 ¦ Wire.read()) / 16384.0;

az = (Wire.read() << 8 ¦ Wire.read()) / 16384.0;

Wire.read(); Wire.read(); // skip temperature

Wire.read(); Wire.read(); // skip gyro X

Wire.read(); Wire.read(); // skip gyro Y

}

void loop() {

float ax, ay, az;

readMPU(ax, ay, az);

accMagnitude = sqrt(ax*ax + ay*ay + az*az);

diff = abs(accMagnitude - prevMagnitude);

prevMagnitude = accMagnitude;

filteredDiff = (0.7 * filteredDiff) + (0.3 * diff);

if (filteredDiff > threshold) {

quakeCounter++;

} else {

if (quakeCounter > 0) quakeCounter--;

}

String status;

if (quakeCounter > 6) {

status = "EARTHQUAKE!";

triggerAlarm();

} else if (quakeCounter > 2) {

status = "WARNING";

} else {

status = "NORMAL";

}

digitalWrite(ledPin, (status == "EARTHQUAKE!") ? HIGH : LOW);

String data = status + " ¦ D:" + String(filteredDiff);

udp.beginPacket(udpAddress, udpPort);

udp.print(data);

udp.endPacket();

lcd.setCursor(0, 0);

lcd.print("Status: ");

lcd.setCursor(8, 0);

lcd.print(status);

lcd.setCursor(0, 1);

lcd.print("D:");

lcd.print(filteredDiff);

lcd.print(" ");

delay(100);

}

void triggerAlarm() {

lcd.setCursor(0, 0);

lcd.print("!!! ALERT !!! ");

lcd.setCursor(0, 1);

lcd.print("EVACUATE NOW ");

for (int i = 0; i < 5; i++) {

digitalWrite(buzzerPin, HIGH);

delay(150);

digitalWrite(buzzerPin, LOW);

delay(150);

}

}

## How the Detection Algorithm Works

### Step 1 — Read raw acceleration

The readMPU() function sends an I2C request to register 0x3B on the MPU6050. It reads 14 bytes: 6 for accelerometer X/Y/Z, 2 for temperature (skipped), and 6 for gyroscope X/Y/Z (skipped). Each accelerometer value is divided by 16384 to convert from the raw 16-bit integer to g-force units.

### Step 2 — Compute acceleration magnitude

accMagnitude = sqrt(ax*ax + ay*ay + az*az);

This gives a single number representing the total vibration force across all three axes. At rest, this will be approximately 1.0g (gravity).

### Step 3 — Compute instantaneous change

diff = abs(accMagnitude - prevMagnitude);

Taking the difference between the current and previous magnitude captures sudden changes in vibration intensity — exactly what an earthquake or shaking produces.

### Step 4 — Apply low-pass filter

filteredDiff = (0.7 * filteredDiff) + (0.3 * diff);

This exponential moving average gives 70% weight to the past and 30% to the new reading. It smooths out single-sample spikes (like bumping the table once) while still responding to sustained shaking.

### Step 5 — Counter-based classification

The quakeCounter increments each loop when filteredDiff exceeds the threshold. It decrements when motion drops. This means a genuine earthquake (sustained shaking over many loops) will push the counter well above 6, while a single bump will not. This eliminates false positives.

## Upload Steps

- Paste the full code into Arduino IDE.

- Update ssid, password, and udpAddress at the top.

- Select your COM port under Tools > Port.

- Click the Upload button (right arrow).

- Wait for Uploading... then Done uploading.

- Open Tools > Serial Monitor at 115200 baud.

- You should see dots as WiFi connects, then: Diff: 0.00  Filtered: 0.00

✔ Upload Success: If the LCD shows WiFi Connected followed by the SeismoGuard status screen, the firmware is working correctly. The Serial Monitor will show live Diff and Filtered values every 100ms.

## Tuning the Sensitivity

| Situation | Recommended threshold value |
| --- | --- |
| Too many false alarms (triggers on footsteps/bumps) | Increase to 1.2 or 1.5 |
| Default starting point | 0.8 (current value) |
| Too insensitive — misses real shaking | Decrease to 0.4 or 0.5 |
| Very sensitive environment / lab use | 0.3 or lower |

# 5.  Database Setup — XAMPP & MySQL

# STEP 9  —  Install and Start XAMPP

- Download XAMPP from https://www.apachefriends.org

- Install it with default settings (install to C:\xampp).

- Open XAMPP Control Panel.

- Click Start next to Apache.

- Click Start next to MySQL.

- Both rows should turn green showing they are running.

⚠ Must be running: Both Apache AND MySQL must be started in XAMPP before any part of the web system works. If either shows red, click Start again or restart XAMPP as Administrator.

# STEP 10  —  Create the Database

- Open your browser and navigate to http://localhost/phpmyadmin

- Click the SQL tab in the top navigation bar.

- Paste the following SQL into the text box:

-- Create the database

CREATE DATABASE IF NOT EXISTS quakesense;

-- Use it

USE quakesense;

-- Create the events table

CREATE TABLE IF NOT EXISTS seismic_data (

id INT AUTO_INCREMENT PRIMARY KEY,

status VARCHAR(20) NOT NULL,

diff_value FLOAT NOT NULL,

recorded_at DATETIME DEFAULT CURRENT_TIMESTAMP

);

- Click the Go button.

- You should see: Your SQL query has been executed successfully.

- The quakesense database now appears in the left sidebar.

## Database Schema Explained

| Column | Data Type | Description |
| --- | --- | --- |
| id | INT AUTO_INCREMENT | Unique ID for each event. Increments automatically on every insert. |
| status | VARCHAR(20) | The alert level: either WARNING or EARTHQUAKE! (NORMAL is never saved). |
| diff_value | FLOAT | The filteredDiff value at the time the event was captured. |
| recorded_at | DATETIME | Timestamp — set automatically by MySQL when the row is inserted. |

ℹ Why not save NORMAL? NORMAL status is filtered out in receiver.py before writing to the database. Saving NORMAL would create thousands of rows per hour of uninteresting data. Only actionable events (WARNING, EARTHQUAKE!) are stored.

## Verifying Your Data

After the system is running, you can inspect the database at any time:

- Go to http://localhost/phpmyadmin

- Click quakesense in the left sidebar.

- Click seismic_data.

- Click Browse to see all records.

- Or run a custom SQL query — example:

-- View latest 20 events

SELECT * FROM seismic_data ORDER BY recorded_at DESC LIMIT 20;

-- Count events by type

SELECT status, COUNT(*) as total FROM seismic_data GROUP BY status;

-- Delete all records (reset)

TRUNCATE TABLE seismic_data;

# 6.  Python UDP Receiver (receiver.py)

# STEP 11  —  Install Python and Dependencies

- Make sure Python 3.11 or later is installed. Check with:

python --version

- Install the MySQL connector library:

pip install mysql-connector-python

⚠ Multiple Python versions: If you have multiple Python versions installed, make sure you run both pip install and the script using the same Python. If pip installs to Python 3.11 but you run the script with Python 3.14, you will get a ModuleNotFoundError. Use python --version to confirm.

# STEP 12  —  Configure receiver.py

Open receiver.py and check these values at the top of the file:

| Variable | Correct Value |
| --- | --- |
| UDP_IP | "0.0.0.0" — listens on all network interfaces (do not change this) |
| UDP_PORT | 4210 — must match the udpPort in your Arduino code |
| DB_HOST | "localhost" |
| DB_USER | "root" |
| DB_PASS | "" (empty string — XAMPP MySQL default has no password) |
| DB_NAME | "quakesense" |

## Full receiver.py Code

import socket

import mysql.connector

from datetime import datetime

# ---- CONFIG ----

UDP_IP = "0.0.0.0"

UDP_PORT = 4210

DB_HOST = "localhost"

DB_USER = "root"

DB_PASS = ""

DB_NAME = "quakesense"

def connect_db():

return mysql.connector.connect(

host=DB_HOST, user=DB_USER,

password=DB_PASS, database=DB_NAME

)

def parse_packet(raw):

try:

parts = raw.strip().split("¦")

status = parts[0].strip()

diff_value = float(parts[1].strip().split(":")[1])

return status, diff_value

except Exception as e:

print(f"[PARSE ERROR] {e} ¦ raw='{raw}'")

return None

def save_to_db(cursor, status, diff_value):

sql = """

INSERT INTO seismic_data (status, diff_value, recorded_at)

VALUES (%s, %s, %s)

"""

cursor.execute(sql, (status, diff_value, datetime.now()))

def main():

sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

sock.bind((UDP_IP, UDP_PORT))

print(f"[*] Listening on port {UDP_PORT}...")

db = connect_db()

cursor = db.cursor()

print(f"[*] Connected to MySQL '{DB_NAME}'")

print("[*] Saving WARNING and EARTHQUAKE! only")

print("-" * 50)

try:

while True:

data, addr = sock.recvfrom(1024)

raw = data.decode("utf-8")

timestamp = datetime.now().strftime("%Y-%m-%d %H:%M:%S")

print(f"[{timestamp}] From {addr[0]} -> {raw}")

result = parse_packet(raw)

if result:

status, diff_value = result

if status in ("WARNING", "EARTHQUAKE!"):

save_to_db(cursor, status, diff_value)

db.commit()

print(f" Saved: {status} Diff={diff_value:.4f}")

else:

print(f" Skipped (NORMAL)")

else:

print(" Skipped (parse failed)")

except KeyboardInterrupt:

print("\n[!] Stopped.")

finally:

cursor.close()

db.close()

sock.close()

if __name__ == "__main__":

main()

# STEP 13  —  Run the Receiver

- Open Command Prompt or PowerShell.

- Navigate to your project folder:

cd C:\xampp\htdocs\project

- Run the script:

python receiver.py

- You should see the startup messages:

[*] Listening on port 4210...

[*] Connected to MySQL 'quakesense'

[*] Saving WARNING and EARTHQUAKE! only

--------------------------------------------------

- Leave this terminal window open. Do not close it while the system is running.

## How Packet Parsing Works

The ESP32 sends UDP packets in this exact format:

NORMAL ¦ D:0.0312

WARNING ¦ D:0.9500

EARTHQUAKE! ¦ D:1.4500

The parse_packet() function splits on the pipe character '|' to get the status on the left and the D:value string on the right. It then splits the right part on ':' to extract the float number. If any step fails, None is returned and the packet is skipped safely.

# 7.  Live Web Dashboard

## File Structure

Create a folder called seismoguard inside your XAMPP web root and place both files there:

C:\xampp\htdocs\seismoguard\

index.html <-- The dashboard UI

api.php <-- The PHP backend (MySQL connector)

# STEP 14  —  Create api.php

This file is the bridge between the database and the dashboard. It runs a MySQL query and returns the results as JSON. Save it as api.php inside the seismoguard folder.

<?php

header("Content-Type: application/json");

header("Access-Control-Allow-Origin: *");

$host = "localhost";

$user = "root";

$pass = "";

$dbname = "quakesense";

$conn = new mysqli($host, $user, $pass, $dbname);

if ($conn->connect_error) {

echo json_encode(["error" => $conn->connect_error]);

exit;

}

$sql = "SELECT id, status, diff_value, recorded_at

FROM seismic_data

ORDER BY recorded_at DESC LIMIT 50";

$result = $conn->query($sql);

$rows = [];

while ($row = $result->fetch_assoc()) { $rows[] = $row; }

$statsSql = "SELECT

COUNT(*) AS total,

SUM(status='WARNING') AS warnings,

SUM(status='EARTHQUAKE!') AS earthquakes

FROM seismic_data";

$statsResult = $conn->query($statsSql);

$stats = $statsResult->fetch_assoc();

echo json_encode(["records" => $rows, "stats" => $stats]);

$conn->close();

?>

# STEP 15  —  Create index.html (Dashboard)

This is the live web dashboard. It uses JavaScript to call api.php every 3 seconds and update the page without a reload. The full code is shown below — save it as index.html in the seismoguard folder.

ℹ Dashboard Features: The dashboard shows: (1) A large status hero with the current alert level, (2) Three stat cards with totals, (3) A live event log table, and (4) A full-screen red flashing border when EARTHQUAKE! is the latest event.

## Dashboard UI Components

| Component | Description |
| --- | --- |
| Status Hero | Large centered text showing NORMAL / WARNING / EARTHQUAKE! with color-coded glow animations. |
| Stat Cards | Three cards: Total Events, Warnings, Earthquakes. Updated every 3 seconds. |
| Event Log Table | Shows latest 50 records with ID, badge, diff value, and timestamp. |
| Alert Overlay | Full-screen red pulsing border that appears when the latest status is EARTHQUAKE! |
| Live Countdown | Shows seconds until next data refresh in the table header. |
| Seismic Wave | Decorative SVG wave line at the bottom of the status hero. |

## Color Coding System

| Status | Hex Color | Visual Behaviour |
| --- | --- | --- |
| NORMAL | #00E5FF (Cyan) | Steady cyan text glow. No animations. |
| WARNING | #FFB300 (Amber) | Amber glow + subtle flicker animation on the text. |
| EARTHQUAKE! | #FF1744 (Red) | Red glow + shake animation + full-screen border pulses red. |

# STEP 16  —  Open the Dashboard

- Confirm XAMPP Apache and MySQL are both started (green).

- Confirm receiver.py is running in its terminal window.

- Confirm the ESP32 is powered and connected to WiFi.

- Open your browser and go to:

http://localhost/seismoguard/

- The dashboard loads and starts refreshing automatically.

- Tap or shake the ESP32 to test — WARNING should appear within a few seconds.

# 8.  Troubleshooting Guide

## Arduino / ESP32 Issues

| Problem | Solution |
| --- | --- |
| LCD shows nothing after wiring | Try changing 0x27 to 0x3F in LiquidCrystal_I2C lcd(0x3F, 16, 2). Some modules use a different I2C address. |
| LiquidCrystal_I2C compile error | Uninstall the current library. In Library Manager, install specifically the version by Frank de Brabander. |
| WiFi Failed shown on LCD | Double-check ssid and password — they are case-sensitive. Confirm the ESP32 and PC are on the same WiFi network. |
| Serial Monitor shows garbage characters | Change the baud rate in Serial Monitor to 115200 to match Serial.begin(115200). |
| Upload fails — port not found | Install the CP2102 or CH340 driver for your ESP32 board. Reconnect the USB cable and retry. |
| udp was not declared error | Make sure WiFiUDP udp; is declared globally at the top of the sketch, and udp.begin(udpPort) is called in setup(). |
| Too many false EARTHQUAKE triggers | Increase threshold from 0.8 to 1.2 or 1.5 in the code. |
| Detector misses real shaking | Decrease threshold from 0.8 to 0.4 or 0.5. |

## Python / receiver.py Issues

| Problem | Solution |
| --- | --- |
| ModuleNotFoundError: No module named mysql | The mysql-connector-python package is not installed for the Python version you are using. Run: pip install mysql-connector-python — then confirm you run the script with the same python that has the package. |
| Cannot connect to MySQL | Confirm MySQL is Started (green) in XAMPP Control Panel. Confirm DB_PASS is empty string and DB_NAME is quakesense. |
| No packets received — script just waits | Check Windows Firewall. Go to Windows Defender Firewall > Advanced Settings > Inbound Rules > New Rule > Port > UDP > 4210 > Allow. |
| UDP packets arrive but parsing fails | Open Serial Monitor on Arduino and check the exact format being sent. It must match: STATUS ¦ D:value with the pipe and space characters. |

## Web Dashboard Issues

| Problem | Solution |
| --- | --- |
| Dashboard page does not load | Confirm Apache is Started in XAMPP. Confirm files are in C:\xampp\htdocs\seismoguard\ not htdocs\project. |
| Dashboard loads but shows no data | Open http://localhost/seismoguard/api.php directly. If it shows a DB error, check api.php credentials. If it shows empty records array, no events are in the DB yet. |
| api.php shows connect_error | MySQL is not running or credentials in api.php do not match XAMPP settings. |
| Data in DB but not in dashboard | Hard-refresh the browser with Ctrl+Shift+R. Check browser console (F12) for JS errors. |
| Only NORMAL showing — no events saved | NORMAL is intentionally not saved. Shake the ESP32 more vigorously to trigger WARNING or adjust the threshold lower. |

## Network / IP Address Issues

| Problem | Solution |
| --- | --- |
| ESP32 sends UDP but PC never receives it | Run ipconfig in Command Prompt and confirm your PC's IPv4 Address. Update udpAddress in the Arduino code to match exactly. |
| IP changes every time router restarts | Set a static IP for your PC in router settings, or set it manually in Windows Network Adapter settings. |
| ESP32 on different network than PC | Both ESP32 and PC must be on the exact same WiFi network and subnet. Using a VPN on the PC can block local UDP traffic. |

# 9.  Full System Startup Checklist

Use this checklist every time you power up and run SeismoGuard. Tick off each item in the Status column as you confirm it.

| # | Status | Action to Confirm |
| --- | --- | --- |
| 1 | [ ] | XAMPP Control Panel > Apache = green (Started) |
| 2 | [ ] | XAMPP Control Panel > MySQL = green (Started) |
| 3 | [ ] | Open terminal > cd C:\xampp\htdocs\project > python receiver.py |
| 4 | [ ] | Terminal confirms: Connected to MySQL database 'quakesense' |
| 5 | [ ] | Terminal confirms: Listening on port 4210 |
| 6 | [ ] | Power on ESP32 via USB or external supply |
| 7 | [ ] | LCD shows: WiFi Connected (then SeismoGuard status screen) |
| 8 | [ ] | Serial Monitor shows: Diff and Filtered values updating |
| 9 | [ ] | Open browser > http://localhost/seismoguard/ > dashboard loads |
| 10 | [ ] | Pulse dot in header is blinking (confirms JS is running) |
| 11 | [ ] | Shake ESP32 gently > WARNING badge appears in dashboard within 3 seconds |
| 12 | [ ] | Check phpMyAdmin > quakesense > seismic_data > new rows added |

✔ System Operational: When all 12 checkpoints pass, SeismoGuard is fully operational. The ESP32 is detecting, the Python receiver is saving, and the dashboard is displaying live data. Any earthquake-level event will trigger the alarm, save to the database, and flash the dashboard red within 3 seconds.

# 10.  Quick Reference Card

## Key URLs

| URL | Purpose |
| --- | --- |
| http://localhost/seismoguard/ | Live SeismoGuard dashboard |
| http://localhost/phpmyadmin | MySQL database management |
| http://localhost/seismoguard/api.php | Raw JSON data endpoint — for debugging |

## Key GPIO Pin Summary

| GPIO Pin | Connected to |
| --- | --- |
| GPIO 2 | LED anode (via 220 ohm resistor) |
| GPIO 13 | Buzzer positive terminal |
| GPIO 21 | SDA — shared by MPU6050 and LCD |
| GPIO 22 | SCL — shared by MPU6050 and LCD |

## I2C Addresses

| Device | I2C Address |
| --- | --- |
| MPU6050 Accelerometer | 0x68 (AD0 connected to GND) |
| LCD 16x2 (I2C backpack) | 0x27 (try 0x3F if LCD is blank) |

## UDP Packet Format

Format: STATUS ¦ D:filteredDiff

Examples:

NORMAL ¦ D:0.0312

WARNING ¦ D:0.9500

EARTHQUAKE! ¦ D:1.4500

Sent from: ESP32

Sent to: Your PC IP address, port 4210

Received: receiver.py listening on 0.0.0.0:4210

## Threshold Quick Guide

| threshold Value | Use When |
| --- | --- |
| 0.3 – 0.5 | Very sensitive — detects minor vibrations |
| 0.8 | Default — balanced for typical indoor use |
| 1.2 – 1.5 | Less sensitive — reduces false alarms from footsteps/bumps |

## Common Commands

# Find your PC's IP address (run in Command Prompt):

ipconfig

# Run receiver.py:

cd C:\xampp\htdocs\project

python receiver.py

# Install mysql connector (if needed):

pip install mysql-connector-python

# Check Python version:

python --version

SeismoGuard  —  Real-Time Seismic Protection System  —  v1.0  —  2026